The deault policies for various security levels are applied as below.

Other application services which are not listed here are blocked if

the firewall is enabled.

Security Level; Low; Medium; High. Service; Port; In; Out; In; Out; In; Out. HTTP(tcp); 80; Yes; Yes; No; Yes; No; Yes. DNS(udp); 53; Yes; Yes; No; Yes; No; Yes. FTP(tcp); 21; Yes; Yes; No; Yes; No; Yes. Telnet(tcp); 23; Yes; Yes; No; Yes; No; Yes. ICMP; N/A; Yes; Yes; Yes; Yes; No; Yes. SMTP(tcp); 25; Yes; Yes; No; Yes; No; Yes. POP3(tcp); 110; Yes; Yes; No; Yes; No; Yes. HTTP-SSL(tcp); 443; Yes; Yes; No; Yes; No; Yes. News-NNTP(tcp); 119; Yes; Yes; No; Yes; No; No. Internet Locator Server(tcp); 389; Yes; Yes; Yes; Yes; No; No. User Location Server(tcp); 522; Yes; Yes; Yes; Yes; No; No. T.120(tcp); 1503; Yes; Yes; Yes; Yes; No; No. H.323 call setup(tcp); 1720; Yes; Yes; Yes; Yes; No; No. Audio call control(tcp); 1731; Yes; Yes; Yes; Yes; No; No. RTP(udp); 5004-65535; No; Yes; No; Yes; No; No. MSN Messager File Transfer(tcp); 6891-6900; Yes; Yes; Yes; Yes; No; No. Remote Messenger Remote Assistance(tcp); 3389; Yes; Yes; Yes; Yes; No; No. MSN Messenger Messaging(tcp); 1863; Yes; Yes; Yes; Yes; No; No. MSN Messenger Voice Comm(tcp/udp); 6901; Yes; Yes; Yes; Yes; No; No. Yahoo! Messenger Webcam(tcp); 5100; Yes; Yes; Yes; Yes; No; No. RealAudio/Video(tcp/udp); 554; Yes; Yes; No; No; No; No. RealAudio/Video(tcp); 7070-7071; Yes; Yes; No; No; No; No. RealAudio/Video(udp); 6770-7170; Yes; Yes; No; No; No; No. MS Media Player(tcp/udp); 1755; Yes; Yes; No; No; No; No. PPTP(tcp/udp); 1723; Yes; Yes; Yes; Yes; No; No. GRE(47); N/A; Yes; Yes; Yes; Yes; No; No. ESP(50); N/A; Yes; Yes; No; No; No; No. AH(51); N/A; Yes; Yes; No; No; No; No. IKE(udp); 500; Yes; Yes; No; No; No; No. ICQ(tcp); 5190; Yes; Yes; Yes; Yes; No; No. ICQ Chat(tcp); 7152; Yes; Yes; Yes; Yes; No; No. (tcp); 411-412; Yes; Yes; No; No; No; No. (tcp); 4998-4999; Yes; Yes; No; No; No; No.

(Yes: allow; No: block)